I =

[  50   50   80   80

50  100  100   80

150  150  100   80

200  200  150  100 ]

Tasks:

- Find the histogram

- Compute the CDF

- Find the histogram equalized image using

s=(L-1)×CDF=255×CDF
1. Find Histogram

List unique values and count them:

| Gray value | 50 | 80 | 100 | 150 | 200 |
| --- | --- | --- | --- | --- | --- |
| Count | 3 | 4 | 4 | 3 | 2 |

Total pixels N=16
For 8-bit image: L=256⇒L-1=255

2. Compute Probability and CDF

Probability p(r)=count16

| r | Count | p(r) | CDF (running sum) |
| --- | --- | --- | --- |
| 50 | 3 | 3/16 = 0.1875 | 0.1875 |
| 80 | 4 | 4/16 = 0.25 | 0.1875 + 0.25 = 0.4375 |
| 100 | 4 | 0.25 | 0.6875 |
| 150 | 3 | 0.1875 | 0.875 |
| 200 | 2 | 0.125 | 1 .0 |

3.Compute New Gray Levels

s=255×CDF

| r | CDF | s = 255×CDF | Rounded |
| --- | --- | --- | --- |
| 50 | 0.1875 | 47.8 | 48 |
| 80 | 0.4375 | 111.6 | 112 |
| 100 | 0.6875 | 175.3 | 175 |
| 150 | 0.875 | 223.1 | 223 |
| 200 | 1.0 | 255 | 255 |

[  50   50   80   80

50  100  100   80

150  150  100   80

200  200  150  100 ]

I_eq =

[  48   48  112  112

48  175  175  112

223  223  175  112

255  255  223  175 ]

Piecewise Linear Contrast Stretching (2D, 8-bit Image)

Given the 3×3 grayscale image:

I =

[  20   60  110

140  170  210

90  130  190 ]

Use the following piecewise transformation:

- If r ≤ 80 → s = r / 2

- If 80 < r ≤ 160 → s = 2(r − 80) + 40

- If r > 160 → s = (r − 160)/2 + 200

- Compute the output value for each pixel

- Write the resulting image

Ans: Apply rule value by value:

For r ≤ 80:

- 20 → 20/2 = 10

- 60 → 60/2 = 30

For 80 < r ≤ 160:

- 90 → 2(90−80)+40 = 2(10)+40 = 60

- 110 → 2(30)+40 = 100

- 130 → 2(50)+40 = 140

- 140 → 2(60)+40 = 160

For r > 160:

- 170 → (170−160)/2 + 200 = 10/2 + 200 = 205

- 190 → (190−160)/2 + 200 = 30/2 + 200 = 215

- 210 → (210−160)/2 + 200 = 50/2 + 200 = 225

🧾 Final Output Image

I_out =

[  10   30  100

160  205  225

60  140  215 ]

Percentile Stretching (2D, 8-bit Image)

Given the 3×4 grayscale image:

I =

[   5   20   30   40

50   60   70   80

90  100  110  240 ]

Assume:

- Ignore the lowest and highest 10% of values (outliers)

- Then stretch the remaining range to 0–255

Tasks:

- Find rlowand rhigh

- Apply percentile stretching

- Write the output image

Answer

Find the percentiles

List all 12 values (sorted):

[ 5, 20, 30, 40, 50, 60, 70, 80, 90, 100, 110, 240 ]

- 10% of 12 ≈ 1 value at the bottom and 1 value at the top

- Ignore:

  - Lowest → 5

  - Highest → 240

So:

- rlow=20

- rhigh=110

2: Define the mapping

- If r≤20→ s=0

- If r≥110→ s=255

- Else:

s=r2011020×255=r2090×255

Step 3: Compute new values (rounded)

- 5 → 0 (below rlow)

- 20 → 0

- 30 → 1090×255≈28

- 40 → 2090×255≈57

- 50 → 3090×255≈85

- 60 → 4090×255≈113

- 70 → 5090×255≈142

- 80 → 6090×255≈170

- 90 → 7090×255≈198

- 100 → 8090×255≈227

- 110 → 255

- 240 → 255 (above rhigh)

Form the output image

Original:

[   5   20   30   40

50   60   70   80

90  100  110  240 ]

After percentile stretching:

I_ps ≈

[   0    0   28   57

85  113  142  170

198  227  255  255 ]