# LAB SHEET  3

25RA611 Computer Vision

S2 Mtech Robotics

Department of Computer Science & Engineering Amrita School of Computing, Amritapuri Campus

Histogram Processing & Image Enhancement

# Topics:

# 1) Histogram plotting

# 2) Contrast stretching (normalization)

# 3) Histogram equalization

# 4) CLAHE (adaptive equalization)

# 5) Image restoration: Denoising + Sharpening

import cv2

import numpy as np

import matplotlib.pyplot as plt

# Helper function: show images easily

def show(title, img, cmap=None):

"""

title: string to show as title

img: image array

cmap: 'gray' for grayscale

"""

plt.figure(figsize=(5,5))

if cmap == 'gray':

plt.imshow(img, cmap='gray', vmin=0, vmax=255)

else:

# If image is BGR (OpenCV), convert to RGB for matplotlib

if len(img.shape) == 3:

plt.imshow(cv2.cvtColor(img, cv2.COLOR_BGR2RGB))

else:

plt.imshow(img)

plt.title(title)

plt.axis('off')

plt.show()

# Helper function: plot histogram for grayscale image

def plot_hist(title, img_gray):

"""

img_gray: grayscale image (0..255)

"""

hist = cv2.calcHist([img_gray], [0], None, [256], [0, 256])

plt.figure(figsize=(6,4))

plt.plot(hist)

plt.title(title)

plt.xlabel("Pixel Intensity (0-255)")

plt.ylabel("Count")

plt.show()

# STEP 1: Load Image

# Option A (recommended): Use OpenCV sample? Not available always.

# Option B: Read your own image from file. Upload one image to Colab first.

# CHANGE THIS PATH if needed

img_path = "/content/sample.jpg"   # <-- upload your image and rename to sample.jpg

img = cv2.imread(img_path)

# cv2.imread returns None if it cannot find the file

if img is None:

print("❌ Image not found. Please upload an image and set correct img_path.")

else:

print("✅ Image loaded:", img.shape)

# Convert to grayscale (needed for histogram-based methods)

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

show("Original (Grayscale)", gray, cmap='gray')

print("Gray image min/max:", gray.min(), gray.max())

# STEP 2: Histogram of Original Image

plot_hist("Histogram - Original Image", gray)

# STEP 3: Contrast Stretching (Normalization)

# Why? If image contrast is low (values in narrow range),

# stretch it so min becomes 0 and max becomes 255.

min_val = np.min(gray)                 # smallest intensity in image

max_val = np.max(gray)                 # largest intensity in image

# Formula: new = (old - min) * 255 / (max - min)

stretched = ((gray - min_val) * 255.0 / (max_val - min_val)).astype(np.uint8)

show("Contrast Stretched Image", stretched, cmap='gray')

plot_hist("Histogram - Contrast Stretched", stretched)

# STEP 4: Histogram Equalization (Global)

# Why? It redistributes intensities to use full range more effectively.

# Works well when whole image is uniformly low-contrast.

equalized = cv2.equalizeHist(gray)

show("Histogram Equalized (Global)", equalized, cmap='gray')

plot_hist("Histogram - Equalized (Global)", equalized)

# STEP 5: CLAHE (Adaptive Histogram Equalization)

# Why? Global equalization can over-brighten or amplify noise.

# CLAHE works on small tiles and limits contrast enhancement.

clahe = cv2.createCLAHE(clipLimit=2.0, tileGridSize=(8,8))

clahe_img = clahe.apply(gray)

show("CLAHE Result", clahe_img, cmap='gray')

plot_hist("Histogram - CLAHE", clahe_img)

# STEP 6: Image Restoration - Add Noise (for demonstration)

# We add noise artificially so students can test denoising.

# If you already have a noisy image, you can skip this part.

np.random.seed(0)                       # makes noise reproducible

noise = np.random.normal(0, 15, gray.shape).astype(np.float32)  # mean=0, std=15

noisy = np.clip(gray.astype(np.float32) + noise, 0, 255).astype(np.uint8)

show("Noisy Image (Artificial Noise Added)", noisy, cmap='gray')

plot_hist("Histogram - Noisy Image", noisy)

# STEP 7: Image Restoration - Denoising

# 7A) Gaussian Blur

# Removes random noise but also blurs edges slightly.

gaussian_denoised = cv2.GaussianBlur(noisy, (5,5), 0)

show("Denoised using Gaussian Blur", gaussian_denoised, cmap='gray')

# 7B) Median Filter

# Very good for salt-and-pepper noise; preserves edges better.

median_denoised = cv2.medianBlur(noisy, 5)

show("Denoised using Median Filter", median_denoised, cmap='gray')

# STEP 8: Image Enhancement - Sharpening (Unsharp Mask)

# Sharpening increases edge clarity.

# Method: original * 1.5  + blurred * (-0.5)

blur = cv2.GaussianBlur(gray, (9,9), 0)                    # blur the image

sharpened = cv2.addWeighted(gray, 1.5, blur, -0.5, 0)      # combine to sharpen

show("Sharpened Image (Unsharp Mask)", sharpened, cmap='gray')

# STEP 9: Comparison View (Optional)

# Show several results together so students can compare visually.

plt.figure(figsize=(14,8))

plt.subplot(2,3,1); plt.imshow(gray, cmap='gray'); plt.title("Original"); plt.axis('off')

plt.subplot(2,3,2); plt.imshow(stretched, cmap='gray'); plt.title("Contrast Stretched"); plt.axis('off')

plt.subplot(2,3,3); plt.imshow(equalized, cmap='gray'); plt.title("Equalized"); plt.axis('off')

plt.subplot(2,3,4); plt.imshow(clahe_img, cmap='gray'); plt.title("CLAHE"); plt.axis('off')

plt.subplot(2,3,5); plt.imshow(median_denoised, cmap='gray'); plt.title("Median Denoised"); plt.axis('off')

plt.subplot(2,3,6); plt.imshow(sharpened, cmap='gray'); plt.title("Sharpened"); plt.axis('off')

plt.tight_layout()

plt.show()