# LAB SHEET  4

25RA611 Computer Vision

S2 Mtech Robotics

Department of Computer Science & Engineering Amrita School of Computing, Amritapuri Campus

Edge Detection Techniques

Topic: Implementation and comparison of Sobel, Prewitt, Canny, LoG, DoG
Course: Computer Vision Lab (Robotics)

1) Aim

To implement and compare common edge detection techniques:

- Sobel

- Prewitt

- Canny

- LoG (Laplacian of Gaussian)

- DoG (Difference of Gaussian)

2) Learning Outcomes

After this lab, students will be able to:

- Convert an image to grayscale and preprocess it

- Compute gradient-based edges (Sobel, Prewitt)

- Detect edges using multi-stage detection (Canny)

- Detect edges using second-derivative methods (LoG, DoG)

- Compare outputs and comment on noise sensitivity and edge thickness

3) Software Requirements

- Python 3.x

- Libraries: numpy, matplotlib, opencv-python, scikit-image

Install (once):

pip install numpy matplotlib opencv-python scikit-image

4) Input Image (Choose any)

Use any one:

- a road/lane image

- indoor objects (cup, chair)

- medical image (optional)

- sample images: coins.png, camera.png from skimage

5) Step-by-Step Procedure + CODE

Step 0: Import Libraries

Output you should see: No output (only imports)

import numpy as np
import matplotlib.pyplot as plt

import cv2

from skimage import data
from skimage.filters import prewitt
from skimage.feature import canny
from skimage.filters import gaussian, laplace

Step 1: Load Image

We’ll use a built-in image so every student gets same output.

Expected output: A grayscale image displayed.

# Option A: Use built-in image (recommended for lab)
img = data.camera()  # already grayscale
img = img.astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(img, cmap='gray')
plt.title("Original Image")
plt.axis('off')
plt.show()

What you will see: A grayscale photo (camera-man image).

Step 2: Add Noise (to test robustness)

Expected output: Noisy image displayed.

np.random.seed(0)
noise = np.random.normal(0, 15, img.shape)  # Gaussian noise
noisy = img.astype(np.float32) + noise
noisy = np.clip(noisy, 0, 255).astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(noisy, cmap='gray')
plt.title("Noisy Image")
plt.axis('off')
plt.show()

What you will see: Same image but grainy/noisy.

Step 3: Smooth the image (preprocessing)

Edge detectors work better after smoothing.

Expected output: Slightly blurred image.

blur = cv2.GaussianBlur(noisy, (5,5), 1.0)

plt.figure(figsize=(5,5))
plt.imshow(blur, cmap='gray')
plt.title("Smoothed (Gaussian Blur)")
plt.axis('off')
plt.show()

What you will see: Noise reduced, image smoother.

Step 4: SOBEL Edge Detection

Sobel uses gradient in x and y direction.

Expected output: Edge map (white edges on black background)

sx = cv2.Sobel(blur, cv2.CV_64F, 1, 0, ksize=3)
sy = cv2.Sobel(blur, cv2.CV_64F, 0, 1, ksize=3)
sobel_mag = np.sqrt(sx**2 + sy**2)
sobel_mag = (sobel_mag / sobel_mag.max() * 255).astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(sobel_mag, cmap='gray')
plt.title("Sobel Edge Map")
plt.axis('off')
plt.show()

You will see: Thick edges, sometimes noisy.

Step 5: PREWITT Edge Detection

Prewitt is similar to Sobel but simpler kernels.

Expected output: Slightly different edge map than Sobel.

prew = prewitt(blur)  # returns float in [0,1] approx
prew_img = (prew / prew.max() * 255).astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(prew_img, cmap='gray')
plt.title("Prewitt Edge Map")
plt.axis('off')
plt.show()

You will see: Edges similar to Sobel, slightly less emphasis.

Step 6: CANNY Edge Detection

Canny is best for clean, thin edges.

Expected output: Thin edges (binary, very clear)

# skimage canny expects normalized float image
blur_float = blur.astype(np.float32) / 255.0

canny_edges = canny(blur_float, sigma=1.2)  # sigma controls smoothing

plt.figure(figsize=(5,5))
plt.imshow(canny_edges, cmap='gray')
plt.title("Canny Edge Map")
plt.axis('off')
plt.show()

You will see: Very thin, clean edges (black-white).

Step 7: LoG (Laplacian of Gaussian)

LoG = Gaussian smoothing + Laplacian (2nd derivative)

Expected output: Edge-like response; may appear double-edge sometimes.

# Smooth then apply Laplacian
g = gaussian(blur_float, sigma=1.2)
log_resp = laplace(g)  # can produce + and - values

# Convert to displayable image
log_disp = np.abs(log_resp)
log_disp = (log_disp / log_disp.max() * 255).astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(log_disp, cmap='gray')
plt.title("LoG Response (|Laplacian(Gaussian)|)")
plt.axis('off')
plt.show()

You will see: Strong responses around edges; sometimes thicker/double.

Step 8: DoG (Difference of Gaussian)

DoG approximates LoG by subtracting two blurred images.

Expected output: Edge response similar to LoG.

g1 = gaussian(blur_float, sigma=1.0)
g2 = gaussian(blur_float, sigma=2.0)
dog = g1 - g2

dog_disp = np.abs(dog)
dog_disp = (dog_disp / dog_disp.max() * 255).astype(np.uint8)

plt.figure(figsize=(5,5))
plt.imshow(dog_disp, cmap='gray')
plt.title("DoG Response |G(sigma=1) - G(sigma=2)|")
plt.axis('off')
plt.show()

You will see: Edges highlighted; similar to LoG.

Step 9: Display All Results Together (Comparison)

Expected output: One figure with all methods side-by-side

fig, ax = plt.subplots(2, 3, figsize=(14, 9))

ax[0,0].imshow(img, cmap='gray'); ax[0,0].set_title("Original"); ax[0,0].axis('off')
ax[0,1].imshow(noisy, cmap='gray'); ax[0,1].set_title("Noisy"); ax[0,1].axis('off')
ax[0,2].imshow(blur, cmap='gray'); ax[0,2].set_title("Smoothed"); ax[0,2].axis('off')

ax[1,0].imshow(sobel_mag, cmap='gray'); ax[1,0].set_title("Sobel"); ax[1,0].axis('off')
ax[1,1].imshow(prew_img, cmap='gray'); ax[1,1].set_title("Prewitt"); ax[1,1].axis('off')
ax[1,2].imshow(canny_edges, cmap='gray'); ax[1,2].set_title("Canny"); ax[1,2].axis('off')

plt.tight_layout()
plt.show()

(Optional: show LoG and DoG in another figure)

fig, ax = plt.subplots(1, 2, figsize=(10, 4))
ax[0].imshow(log_disp, cmap='gray'); ax[0].set_title("LoG"); ax[0].axis('off')
ax[1].imshow(dog_disp, cmap='gray'); ax[1].set_title("DoG"); ax[1].axis('off')
plt.tight_layout()
plt.show()

Answer these

- Why do we apply Gaussian smoothing before edge detection?

- What is the difference between first derivative (Sobel) and second derivative (LoG)?

- Why is Canny called a “multi-stage” edge detector?

- Which method gives the thinnest edges and why?

- What happens to Sobel output when noise increases?

Top of Form

Bottom of Form